DMXConfig

Datamax Printer Configuration Utility Commands

System Settings

Back After Print1 – When the present distance is set with the GPIO enabled, this command determines the timing of the label back up. When enabled, the printer immediately backs up the label after the applicator-issued start of print signal is received, resulting in faster throughput. If disabled, the printer will not initiate repositioning until the next label is ready to print (may help prevent the curling of the label edge).

Buzzer Enable3 – This command controls the audible signaling device that acknowledges User Interface entries and, if enabled, sounds printer warning and fault conditions.

Default Module1 – This command, typically used prior to the loading of PCL-4 bit-mapped fonts (see Font Loading Commands), is designed to allow the user to select between modules when downloading information. The default module is one of the following:

- The first alpha designator of the existing modules if item 2 has not occurred.

- The module selected by this command.

Double Byte Symbol Set1 – This command selects the font symbol set for Unicode fonts such as Chinese or Japanese. The selected symbol set remains active until another symbol set is selected. Option dependant. Not all symbol sets can be used with all fonts.

DPL Emulation1,2 – This command enables the <STX>O and <STX>f print positioning commands to allow backward compatibility with label formats that were designed for the Allegro® Prodigy®, and Prodigy Plus®.

Exact Time Print2 – Will cause the label printing to format and print so that the actual printer time shows on the label when printing a time field.

Fault Handling1 – When a fault condition (ribbon out, media out, etc.) is detected, this command determines the level of intervention and the disposition of the label being printed at the time the fault occurred.

| Units / Interpretation | Definition |  |
| --- | --- | --- |
| Interaction Level | Standard | Printing stops and a fault message is displayed. After the problem is corrected, the FEED Key must be pressed to clear the fault. The label in process is reprinted. |
|  | Void and Retry | Depending upon the RETRY COUNT, one of the following actions when faulted: · If the Retry Count setting has not been exceeded, ‘VOID’ is printed on the label in process and reprinting occurs automatically; · If the Retry Count has been exceeded, printing stops and a fault message is displayed. After the problem is corrected, the FEED Key must be pressed to clear the fault. The label in process is reprinted; or, · If the CANCEL Key is pressed the operator now has the option of canceling the reprint: To allow the reprint, press the ESCAPE Key or to cancel the reprint, press the ENTER Key (the operator now has the option of canceling the entire label batch by pressing the ENTER Key again.) þ Note: VOID will not be printed when insufficient space exists for the height of the text (see VOID DISTANCE, below) or if the fault occurred after the entire label was completed (i.e., when the label reaches its Present Distance at or above the TOF). |
|  | No Reprint | Printing stops and a fault message is displayed. After the problem is corrected, the FEED Key must be pressed to clear the fault. The label in process is not reprinted. |
| Void Distance | .10 - 2.00 | Sets the distance to backup the faulted label to print ‘VOID’ on its trailing edge, which also indirectly establishes the font size of the void message. |
| Retry Count | 0 - 3 | Establishes the number of times the printer will attempt to reprint a label. If the last label printed in this count has been voided, the printer will stop and display a fault message. |

Font Emulation1 / Font Substitution 2 – This command allows font substitution for CG TIMES, Font ID SA0, or a downloaded font, Font ID S50, in place of the system bitmap fonts 0 – 6 or the Scalable Fonts S00, S01 and SA0 – SA3.

Format Attribute1 –

| Attribute | Description | Example |
| --- | --- | --- |
| XOR Mode | In this mode, the region where text strings, images or bar codes intersect will not be printed. (An odd number of overlapping objects will print.) |  |
| Transparent Mode | This is the default mode; the intersecting regions of text strings, images, and bar codes will print, allowing the user to print fields on top of one another. |  |
| Opaque Mode | Interacting text is obliterated by the text formatted last. Each character cell is treated as opaque. This mode is effective only in rotation 1. See Record Structure Types. |  |
| Inverse Mode | This mode allows inverse (white on black) printing; (e.g., a proportionally sized border and background are printed similar to photographic negative). If text or image fields overlap in this mode, the effect will be similar to the XOR mode. |  |

Head Bias3 – This command instructs the printer to switch the dot zero position: as viewed from the printer’s front panel (or label output side) – when dot zero occupies the left-most location on the printhead then printing is left justified; when dot zero occupies the right-most location, printing is right justified.

Imaging Mode1 – This command instructs the printer whether to pre-image the label format:

| Units / Interpretation | Definition |
| --- | --- |
| Multiple Label | The printer images multiple labels as memory permits, achieving the fastest throughput; however, if time-stamping, the time will reflect the moment the label is imaged rather than when actually printed. |
| Single Label | The printer images the next label only after the previous label has been successfully printed. Single processing provides time-stamps that are more accurate, but it slows label throughput time. |

þ Note: This selection can affect the accuracy of time-stamped labels and label throughput.

Input Mode1,2 - This command defines the type of printer language: Standard DPL processing or Line Mode which exacts data terminated by a carriage return to be inserted in a DPL template and printed.

Internal Module1,2  – This command sets the number of 1K blocks (or 4K blocks for non-display models allocated for the internal RAM module. This command is dependent on the Scalable font cache command. These two commands split the available memory. Available memory is dependent upon printer model. If the combined values exceed available memory both values will be set to factory defaults.

Label Rotation – Label will print rotated by 180 degrees.

Menu Language1  – This command selects the language in which the menu system messages and configuration label are shown. Only languages that are resident will be available.

No Reprint  – Will not reprint label after fault condition is cleared.

Pause Mode1 – This command causes the printer to pause after printing each label. It is intended for use with the peel mechanism or tear bar when the Present Sensor option is not installed. After removing the printed label, the PAUSE Key must be pushed in order to print the next label. (The printer must be reset to clear this command.)

Peel Mode1 – This command, when enabled, specifies that a Feed operation be prevented when the label is presented and not removed, or if the printer is to wait for the GPIO start of print signal.

Scalable Cache1,2 – This command sets the number of 1K blocks (or 4K blocks for non-display models) allocated for the scaleable font engine. This command is dependent on the Internal Module command. These two commands split the available memory. Available memory is dependent upon printer model. If the combined values exceed available memory both values will be set to factory defaults.

Single Byte Symbol Set1,2  – This command selects the scalable font symbol set. The selected symbol set remains active until another symbol set is selected. Option dependant. Not all symbol sets can be used with all fonts.

SOP Emulation1 – This command enables the <STX>O and <STX>f print positioning commands to allow backward compatibility with label formats designed for other printers.

Units of Measure1 – Printer will interpret distance values based on this setting, inches when set for Imperial and millimeters when set to Metric.

Print Control

Column Adjust1 – This command fine-tunes the Column Offset setting by shifting both the horizontal start of print position and the Label Width termination point to the right in dots to compensate for slight mechanical differences sometimes evident if multiple printers share label formats.

Column Offset1 - This command allows horizontal adjustment of the point where printing begins. The printer is instructed to print label formats nnnn units to the right of the position that the format specifies. This feature is useful when a single format is to be printed on labels containing preprinted information.

Darkness1 – This command controls the printhead strobe timing to fine-tune the HEAT setting.

Feed Speed1 - Controls speed of paper movement when the Feed key is pressed or the printer receives a Feed command.

Print Contrast1 - Adjusts edge definition when printing.

Heat1,2  - Controls the amount of heat applied to the label. Controls image quality. Adjusted based on label and ribbon materials.

Present Adjust1 - Fine-tunes the Present Distance.

Present Distance1,2 - Distance paper is advanced beyond the printhead to allow label removal, peel operation, or cut amount.

Print Contrast1 - Fine-tunes print quality.

Print Speed1 - Controls speed of paper movement during printing.

Reverse Speed1 - Controls speed of paper movement when the label is backing up.

Row Adjust1 - Adjusts the starting print position on the label. Allows for adjustment when the same format is used on different printers due to possible slight mechanical differences.

Row Offset1 - Adjusts the starting print position on the label. Allows for adjustment when the same format is used on different printers due to possible slight mechanical differences.

Slew Speed1 - Controls speed of paper movement during printing when nothing is printed in an area. (White space)

SOP Adjust2 – This command sets the start of print (SOP) location, relative to the top of form.

Stop Location2 - This command sets the label stop location after printing.

Media Settings

Continuous Label Length1,2 - Paper length when the printer is set to use continuous label stock. See Sensor Type setting.

Label Width1,2 - Print width of the label. This value is dependent on printhead width.

Maximum Label Length1 - This instructs the printer move media this distance in search of the top-of-form (label edge, notch, black mark, etc.) before declaring a paper fault. A paper fault condition can occur if this setting is too close (within 0.1 inch [2.54 mm]) to the physical length of the label. Therefore, it is a good practice to set this command to 2.5 to 3 times the actual label length used. The minimum value should be at least 5” (127 mm)

Media Type1,2 – This selects the printing method: Direct Thermal for use with heat sensitive media or Thermal Transfer for use with media requiring a ribbon to create an image.

Paper Out Distance1 – This sets the length of travel before an out of stock condition is declared.

Ribbon Low Diameter1  – This sets the threshold for a low ribbon indication.

Sensor Type1,2 - Determines whether the printer looks for label gap, reflective mark, or uses continuous label length to determine print length.

Communications

CNTRL-Codes (Data) 1,2 - When enabled allows printer to act on control codes included in the data portion of label format field.

Control Codes1,2 – This command, depending upon printer type, allows a change to the prefix of the software commands interpreted by the printer:

| Units / Interpretation | Definition |
| --- | --- |
| Standard Codes | Hex 01 = SOH command; Hex 02 = STX command; count-by = ^; Hex 1B = ESC; Hex 0x0D = Carriage Return |
| Alternate Codes | Hex 5E = SOH command; Hex 7E = STX command; count-by = @; Hex 1B = ESC; Hex 0x0D = Carriage Return |
| Alternate Codes 2 | Hex 5E = SOH command; Hex 7E = STX command; count-by = @; Hex 1B = ESC; Hex 0x7C = Carriage Return |

ESC Sequences1 – This command allows data containing invalid ESC control code sequences to be processed (helpful because some systems send a “banner” to the printer). When set to ‘Disabled,’ ESC sequences are ignored and the data is processed. Bitmapped font downloads are disabled in this mode.

Feedback Mode1 - This command enables the feedback ASCII hex characters to be returned from the printer following specific events after each completed batch of labels when using serial communications. The default value is ‘Off’.

| Event | Return Characters |
| --- | --- |
| Invalid character | 0x07 ( BEL ) |
| Label printed | 0x1E ( RS ) |
| End of batch | 0x1F ( US ) |

Heat Command1 – This command causes the printer to disable the DPL Heat command; instead, the Heat value is controlled via the menu setting.

Host Timeout1 – This command controls the number of seconds a communications port must be idle before the printer may process data from a different port. The value is also used to “timeout” an image / label format download (i.e., if, at any time, data flow stops before a complete label format is received, the data will be ignored).

Max Length1 – When enabled printer will ignore Max label length commands.

Symbol Set Commands1,2 – When enabled/on printer will ignore <STX>yS and yS commands.

Speed Commands1 – This command causes the printer to disable the DPL speed commands (Print, Feed, Slew, and Reverse); instead, the speed values are controlled via the menu setting.

<STX>V1 – When enabled printer will ignore <STX>V settings.

Parallel Port Direction1 – This controls data flow directions for the parallel port(s).

Serial Port1,2 – This configures the serial communication port(s).

TOF Sensing1 - Determines if TOF sense commands are acted on.

Sensor Calibration - Modifying these values can cause label sensing and/or print positioning problems.

Empty Sensor Level1 - Sets threshold value for the ‘Empty’ media sensor parameter.

Gap Sensor Level1 - Sets threshold value for the media sensor’s gap parameter.

Label Alignment2 – This command prevents labels with lengths that are less than the distance between the printhead and the Media Sensor from being wasted at power-up. See the appropriate Operator’s Manual for detailed information.

| Media Type, Operation Mode | Label Alignment Setting | Description |
| --- | --- | --- |
| Continuous stock; 6.5” or greater die-cut, notched and reflective stocks; multiple form lengths | Disabled | At power-up, printing will begin at the current label position without any alignment (unless the RTC option is installed). |
| 6.5” or less die-cut, notched and reflective stocks | Automatic | Use for easy label length changes, and to automatically set the maximum label length. Press and hold the FEED Key four seconds and the printer will automatically measure the label length. Once loaded and aligned subsequent power-ups will result in no wasted labels. |
|  | Enabled | Use for constant label lengths. Specify the Label Alignment Length using the <STX>KcAL command or the Setup Menu. Press and hold the FEED Key four seconds, or toggle printer power, then the printer will align with the specified length. |
| 6.5” or less die-cut, notched and reflective stocks with the Present Sensor enabled | Automatic | This mode of operation requires a Label Alignment Length or that Label Alignment be set to AUTO. If either of these conditions is not met, the printer will not perform the alignment. |

Alignment Length2 – Critical for small labels when ‘Label Alignment’ is set to ‘Yes,’ this command allows a length, measured from leading edge to leading edge of two successive labels to be entered. The measured length must be provided to the nearest hundredth of an inch. For very small labels, errors as small as 0.01” can result in noticeable print variations on the labels between the media sensor and the printhead. The number of labels that can be fit between the Media Sensor and the printhead will magnify any error in label alignment length. Errors are more favorable on the low side than on the high side.

Mark Sensor Level1 - Sets threshold value for the media sensor’s mark parameter.

OOS Maxvolt2 - Sets the maximum sensor reading that can indicate Out of Stock with the sensor set to 'EDGE' and the sensor gain set high.

OOS Maxvolt Reflective2 - Sets the maximum sensor reading that can indicate Out of Stock with the sensor set to 'MARK' and the sensor gain set high.

Paper Sensor Level1 - Sets threshold value for the media sensor’s paper parameter.

Refl Paper Level1 - Sets threshold value for the media sensor’s reflective parameter.

Refl Sensor Gain1 - Gain value used when set for reflective sensing.

Reflective TOF Low2 – Sets the minimum sensor reading for mark (Reflective). If this value is set to 0 the setting is not used.

Reflective TOF Delta2 – Sets the minimum change in the sensor reading required to recognize a "Paper to Mark" transition.

Reflective TOF Gain2 – Sets the value to be used for the voltage output to the TOF gain circuit.

Trans Sensor Gain1 - Gain value used when set for gap (edge) sensing.

TOF Low2 – Sets the minimum sensor reading for paper (EDGE). If this value is set to 0 the setting is not used.

TOF Delta2 – Sets the minimum change in the sensor reading required to recognize a "Paper to Gap" transition.

TOF Gain2 – Sets the value to be used for the voltage output to the TOF gain circuit.

Printer Options

Cutter Enable1 - Auto will detect whether cutter is installed and enable it if found. Enable will enable cutter even if not installed - will cause cutter fault. Disable turns off cutter if installed.

GPIO Backup Label1,2 – This command determines the timing of the label back up when the GPIO 1 option is installed and enabled. When enabled, the interface supports an input signal that allows the printer to back up a label once it has been presented.

GPIO Enable1 – This command is used to interface the printer to external controlling devices (see Appendix J):

| Units / Interpretation | Definition |
| --- | --- |
| Applicator | Enables the GPIO for a label applicator. |
| Verifier | Enables the GPIO for a bar code verifier. |
| Disabled | Disables the GPIO Port. |

GPIO End of Print1 – This command defines the programmable signal output that signifies the End of Print (EOP) process:

| Units / Interpretation | Definition |
| --- | --- |
| Active Low | Outputs a logic low (zero) following printing. |
| Active High | Outputs logic high (one) following printing. |
| Low Pulse | Outputs a low pulse (approximately 30 milliseconds long) following printing. |
| High Pulse | Outputs a high pulse (approximately 30 milliseconds long) following printing. |

GPIO Ribbon Low Signal1 – This command sets the signal output that signifies the Ribbon Low Diameter condition for the optional GPIO card.

GPIO Start of Print1 – This command defines the programmable signal input that controls the Start of Print (SOP) process:

| Units / Interpretation | Definition |
| --- | --- |
| Active Low | SOP signal must go low for at least 50 milliseconds to initiate printing. |
| Active High | SOP signal must go high for at least 50 milliseconds to initiate printing. |

Present Sensor Enable1,2 – Auto will detect whether present sensor is installed and enable it if found. Enable will enable present sensor even if not installed - causes present sensor fault.  Disable turns off present sensor if installed.

Retract Delay1 – Controls the time before the label retracts after the previous label has finished printing.

Ribbon Saver Enable1 - This command enables the operation of the optional Ribbon Saver. It is the only command used to control the Ribbon Saver. Its operation is continuous when enabled. The printer must be set to thermal transfer (ribbon) printing mode then, during operation, the Ribbon Saver engages automatically, lifting when the minimum amount of label white space is exceeded.

Scanner Configuration1 – This command configures the optional linear scanner as follows:

| Value | Range / Interpretation |  |  |
| --- | --- | --- | --- |
| Scanner Barcode Count (number of codes to be read): 00 – 99 (00 = Auto [variable] mode) þ Note: Do not use Auto Mode with bitmapped codes or codes with certain addendums; see Appendix F. Scanner min readable height (sets the vertical distance of the code that must have identical reads): 1 - 1/16 2 - 2/16 3 - 3/16 4 - 1/4 5 - 1/2 0 - Disabled (defaults to Redundancy Level 2x) þ Note: The selected distance should not exceed 50% of the measured bar code height Scanner mode (enables scanner detection by the printer): A - auto (automatically senses presence) E - enabled (presence must be detected or a fault is generated) D - disabled (scanner disabled) Barcodes (specifies the bar code, two digits) followed by ‘Y’ to enable or ‘N’ to disable the code 01 - CODE 39 02 - IATA 03 - CODABAR 04 - INTERLEAVED 2 OF 5 05 - INDUSTRIAL 2 OF 5 06 - CODE 93 07 - CODE 128 08 - MSI/PLESSEY 09 - EAN (13/8) 10 - EAN (13/8)+2 11 - EAN (13/8)+5 12 - UPC (A/E) 13 - UPC (A/E)+2 14 - UPC (A/E)+5 þ Note: To maximize throughput and decoding integrity enable only those symbologies that will be read. |  |  |  |
| Redundancy level (alternative method to ensure data integrity where the selected level sets a number of consecutive, identical decodes required to pass the bar code; if a different value is read then the count is restarted). þ Note: Depending upon the print speed, higher levels may cause erroneous failures when scanning multiple or small bar codes. |  |  |  |
| 0 - Auto Mode 1 - read bar code 1X 2 - read bar code 2X 3 - read bar code 3X 4 - read bar code 4X 5 - read bar code 5X |  | 6 - read bar code 6X 7 - read bar code 8X 8 - read bar code 10X 9 - read bar code 12X A - read bar code 15X B - read bar code 20X | C - read bar code 25X D - read bar code 30X E - read bar code 35X F - read bar code 40X G - read bar code 45X |

Verifier Equip – This command enables the GPIO for a bar code verifier.

Notes:

1.	Applies to I, W, A, and M-Class display models.

2.	Applies to E and M-4206.

3.	Applies to A-Class only.