# Registry Keys in XP Universal Driver package

Registry Keys in XP Universal Driver package	1

1	Introduction	2

2	Device Specific Registry Keys	2

2.1	Registry keys for serial	2

2.1.1	Touch Mode	3

2.1.2	Serial Port Configuration	5

2.1.3	Enable / Disable Touch	6

2.1.4	Beep on Touch	7

2.1.5	Drag Delay	8

2.1.6	Full Screen Bounding Rectangle and Bounding Mode	9

2.1.7	Calibration	10

2.1.8	Edge of touchscreen cursor acceleration	12

2.1.9	Special Features	13

2.1.9.1	Clipping Rectangles	13

2.1.9.2	Smartset commands issued during driver startup	14

2.1.9.3	Button Sequence	15

2.1.9.4	Reserved	16

2.2	Registry keys for USB device	17

2.2.1	Registry keys for USB	18

2.2.1.1	Enable / Disable Touch	18

2.2.2	Full Screen Bounding Rectangle and Bounding Mode	19

2.2.2.1	Calibration	20

2.2.3	Edge of touchscreen cursor acceleration	22

2.2.4	Special Features	23

2.2.4.1	Clipping Rectangles	23

2.2.4.2	Smartset commands issued during driver startup	24

2.2.4.3	Reserved	24

2.2.5	Registry keys for HID	25

2.2.5.1	Touch Mode	25

2.2.5.2	Beep On Touch	27

2.2.5.3	Drag Delay	28

2.2.6	Special Features	29

2.2.6.1	Button Sequence	29

3	User Specific Registry Keys	30

# Introduction

This document explains the use of all the registry keys used by the “XP Universal Driver package”.

In order for device specific changes to be effective the device must be restarted (i.e. disable the device and then enable it) in Device Manager or the system must be rebooted.

For user specific changes to become effective it is necessary to log off and log back in again.

# Device Specific Registry Keys

## Registry keys for serial

Device Parameters for Elo serial touchscreen are located in subkeys of

HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Enum\SERENUM\

ELOSERIAL\*******\Device Parameters key.

The “******” is the id used by the Windows system.

### Touch Mode

Name: 		MouseMode

Data Type: 		DWORD (REG_DWORD)

Explanation:

Touchscreen mode.

Possible Values:

Default is 6.

0: Click on touch.

1: Click on release.

6: Mouse emulation.

Name: 		EnableQuickTouch

Data Type: 		DWORD (REG_DWORD)

Explanation:

Enables / disables quick touch mode.

Possible Values:

Default is 0.

0: Disabled

1: Enabled

Name: 		QuickTouchDx

Data Type: 		DWORD (REG_DWORD)

Explanation:

X distance in Windows virtual coordinates outside which quick touch is enabled. Within this distance two quick touches are interpreted as regular touches.

Name: 		QuickTouchDy

Data Type: 		DWORD (REG_DWORD)

Explanation:

Y distance in Windows virtual coordinates outside which quick touch is enabled. Within this distance two quick touches are interpreted as regular touches.

Name: 		UntouchDelay

Data Type: 		DWORD (REG_DWORD)

Explanation:

“UntouchDelay” defines the time in seconds for the timeout. If there is constant touch at the same location a Button Up event will be generated automatically after the timeout period expires.

Possible Values:

Default value is 10 seconds.

Name: 		SwapButton

Data Type: 		DWORD (REG_DWORD)

Explanation:

Swaps the mouse left and right buttons. The buttons remain swapped for the count of touches this value is set to. It remains swapped until this count is decremented to 0. After that, the buttons are automatically swapped back to their original state.

Possible Values:

By default this key is not present in the registry. When the DLL API feature is used the mouse buttons are swapped and this value is added.

Name: 		LeftHandedUser

Data Type: 		DWORD (REG_DWORD)

Explanation:

This is applicable for left-handed mouse users.

Possible Values:

Default is 0

0: Disabled

1: Enabled

Name: 		RightButtonActive

Data Type: 		DWORD (REG_DWORD)

Explanation:

Indicates that the right button application, should be run at user logon for this screen.

Possible Values:

1: Run right button at user logon.

2: Do not run right button for this screen at user logon.

### Serial Port Configuration

Name: 		PortFriendlyName

Data Type: 		String (REG_SZ)

Explanation:

PortFriendlyName identifies the serial port the touchscreen is connected to. The friendly name is the COM port identifier string used by Device Manager and can be found by expanding the Ports node of the Device Manager hardware tree.

To change the COM port associated with a touchscreen, replace the existing value in

HKLM\System\CurrentControlSet\Services\EloTOuchscreen\LegacyPorts\**\FriendlyName

with the friendly name of the desired COM port and restart the system.

Possible Values:

Communications Port (COM1)

Communications Port (COM2)

Name: 		BaudRate

Data Type: 		DWORD (REG_DWORD)

Explanation:

Baud rate used for serial port communication. Controller must be externally configured to support any baudrate other than 9600.

Possible Values:

Default is 9600.

Allowed: 1200, 2400, 4800, 9600.

Name: 		HardwareHandshaking

Data Type: 		DWORD (REG_DWORD)

Explanation:

This key is used only in case of serial cables which are built using 2 wires and do not support hardware handshake.

Possible Values:

By default this key is not present in the registry. It has to be explicitly created.

By default hardware handshaking is always turned on. To turn it off, explicitly set this key to 0.

### Enable / Disable Touch

Name: 		PacketEnable

Data Type: 		DWORD (REG_DWORD)

Explanation:

Touch is enabled / disabled depending on this value.

Possible Values:

1: Enabled

2: Disabled

3: Reserved

### Beep on Touch

Name: 		BeepFlag

Data Type: 		DWORD (REG_DWORD)

Explanation:

Beep is enabled / disabled depending on this value.

Possible Values:

0: Disabled

1: Enabled

Name: 		BeepTime

Data Type: 		DWORD (REG_DWORD)

Explanation:

Defines the duration of beep on touch in milliseconds.

Possible Values:

Default is 0x0014.

Name: 		BeepFreq

Data Type: 		DWORD (REG_DWORD)

Explanation:

Frequency of beep on touch in Hz.

Possible Values:

Default is 0x00300.

### Drag Delay

Name: 		DefaultDragDelay

Data Type: 		DWORD (REG_DWORD)

Explanation:

Drag delay value in milliseconds.

Possible Values:

Default is 20.

Name: 		MinDragDelay

Data Type: 		DWORD (REG_DWORD)

Explanation:

Minimum drag delay value on the scale in Control Panel in milliseconds. Reserved for use by control panel and driver.

Name: 		MaxDragDelay

Data Type: 		DWORD (REG_DWORD)

Explanation:

Maximum drag delay value on the scale in Control Panel in milliseconds. Reserved for use by control panel and driver.

### Full Screen Bounding Rectangle and Bounding Mode

Name: 		FullScreenVmode

Data Type: 		DWORD (REG_DWORD)

Explanation:

Defines the mode for the touchscreen boundary in the virtual desktop mode.

Possible Values:

Default is 2.

0: Disable virtual desktop. Touch on all screens will reflect on the primary monitor.

1: Enable virtual desktop. In this mode no individual screen bounds are used for the monitor. Touches toward the edge may generate clicks on adjacent monitors.

2: Enable virtual desktop. Screen bounds are enabled for each monitor. Touches on the screen always generate clicks on the associated monitor. A touch outside the bounding rectangle will cause the cursor to move to a point along the boundary that is nearest to the point of touch.

3: Enable virtual desktop. Screen bounds are enabled for each monitor. Touches on the screen always generate clicks on the associated monitor. Touches outside the bounding rectangle are not sent to the system.

Name: 		FullScreenBounds

Data Type: 		Array of DWORDs (REG_BINARY)

Explanation:

Defines the full screen bounding rectangle in Windows virtual coordinates for the screen.

### Calibration

The following defines the calibration data used to convert touches from the Elo coordinate system to Windows virtual coordinate system.

The driver uses the following equation to convert a raw touch coordinate point from Elo coordinate system to the Windows screen coordinate system.

Calibration equation is

Xcal = a + m*Xuncal,

where,

Xuncal, is in Elo coordinate system

Xcal, is in Windows coordinate system

m = X_ScrDx / X_EloDx

"a", is the X offset value entry

X_ScrDx = distance between targets in virtual coordinates

X_EloDx = distance between targets in Elo coordinates.

Name: 	X_EloDx, X_ScrDx, X_Offset, Y_EloDx, Y_ScrDx, Y_Offset, Z_EloDx, Z_ScrDx, Z_Offset

Data Type: 		DWORD (REG_DWORD)

Explanation:

Data required by calibration. Use the equation above to calculate these values.

Name: 	SwapXY

Data Type: 	DWORD (REG_DWORD)

Explanation:

Specifies if the touchscreen is rotated 90 degrees or 270 degrees.

Possible Values:

Set to 1 if touchscreen is rotated 90 degrees or 270 degrees else set to 0.

Name: 	WindowsMonitorNumber

Data Type: 	DWORD (REG_DWORD)

Explanation:

Windows monitor number.

Possible Values:

Windows monitor numbers are 1 based.

Name: 	X_Resolution

Data Type: 	DWORD (REG_DWORD)

Explanation:

Screen width in pixels.

Name: 	Y_Resolution

Data Type: 	DWORD (REG_DWORD)

Explanation:

Screen height in pixels.

Name: 	xVirtScrSize

Data Type: 	DWORD (REG_DWORD)

Explanation:

Width in pixels of the Windows virtual screen.

Name: 	yVirtScrSize

Data Type: 	DWORD (REG_DWORD)

Explanation:

Height in pixels of the Windows virtual screen.

Name: 	xVirtScrCoord

Data Type: 	DWORD (REG_DWORD)

Explanation:

Top left X coordinate of the Windows virtual screen.

Name: 	yVirtScrCoord

Data Type: 	DWORD (REG_DWORD)

Explanation:

Top left Y coordinate of the Windows virtual screen.

Name: 	xMonLocn

Data Type: 	DWORD (REG_DWORD)

Explanation:

Monitor location (X) for this monitor on the virtual desktop.

Name: 	yMonLocn

Data Type: 	DWORD (REG_DWORD)

Explanation:

Monitor location (Y) for this monitor on the virtual desktop.

### Edge of touchscreen cursor acceleration

Name: 		EdgeAccelerationEnable

Data Type: 		DWORD (REG_DWORD)

Explanation:

Enables / disables acceleration.

Possible Values:

Default is 0.

0: Disable acceleration.

1: Enable acceleration.

Name: 		EdgeAccelerationBounds

Data Type: 		Array of DWORDS (REG_BINARY)

Explanation:

Rectangle bounds (XMin, YMin, XMax, YMax) beyond which cursor movement is accelerated. Within the defined rectangle touch is interpreted normally. XMin, YMin, XMax, YMax are defined in Windows virtual screen coordinates.

Name: 		EdgeAccelerationScale

Data Type: 		DWORD (REG_DWORD)

Explanation:

Acceleration value.

Possible Values:

Default is unused.

It can be anywhere in the range of 0-100. 10 indicates no acceleration.

### Special Features

#### Clipping Rectangles

Name: 		VirtualDeskMode

Data Type: 		DWORD (REG_DWORD)

Explanation:

Defines the mode for the clipping rectangles for the screens. Every screen can have a maximum of 64 rectangles overlapping or disjoint.

Possible Values:

Default is not used.

2: Clipping at rectangle bounds is enabled. A touch outside the bounding rectangle will cause the cursor to move to a point along the bounding rectangle that is nearest to the point of touch.

3: Clipping at rectangle bounds is enabled. Touches outside the bounding rectangle are not sent to the system.

Name: 		ExclusionFlag

Data Type: 		DWORD (REG_DWORD)

Explanation:

Defines if touch is enabled within the rectangles or outside the defined rectangles.

Possible Values:

Default is 0.

0: Inside the rectangle.

1: Outside the rectangle.

Name: 		Num_Bounds

Data Type: 		DWORD (REG_DWORD)

Explanation:

Count of rectangles defined in “ClippingBounds” array.

Possible Values:

Default is 0.

0: Minimum Rectangles.

64: Maximum Rectangles.

Name: 		ClippingBounds

Data Type: 		Array of DWORDs (REG_BINARY)

Explanation:

Rectangle boundary values defined for clipping on the screen.

#### Smartset commands issued during driver startup

Name: 		SmartSetCommands

Data Type: 		Array of CHARs (REG_BINARY)

Explanation:

This is used for initializing the controller at start time. If this key is present in the registry then the default initialization in not used. This is an array of smartset packets, where each packet is 8 characters. The driver reads 8 chars in a loop till the end and sends them to the controller at initialization. Please refer to the smartset manual for smartset commands.

Possible Values:

By default this key is not present in the registry. It has to be explicitly created.

#### Button Sequence

Use the EloBtnSeq application to set these values.

Name: 		Buttons_Touch

Data Type: 		Array of WORDs  (REG_BINARY)

Explanation:

Defines the sequence of mouse button events generated when user touches the screen (corresponding to Button Down).

Name: 		Buttons_Move

Data Type: 		Array of WORDs  (REG_BINARY)

Explanation:

Defines the sequence of mouse button events generated on Drag (Move).

Name: 		Buttons_UnTouch

Data Type: 		Array of WORDs  (REG_BINARY)

Explanation:

Defines the sequence of mouse button events generated when user lifts his finger (Release / Button Up).

#### Reserved

Name: 		ControllerType

Data Type: 		DWORD (REG_DWORD)

Explanation:

This is reserved for internal use.

## Registry keys for USB device

Device Parameters for USB touchscreen are located in the sub keys of the HID key and the USB key.

HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Enum\USB\Vid_04e7&Pid_???\*******\Device Parameters key for USB.

HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Enum\HID\Vid_04e7&Pid_???\*******\Device Parameters key for HID.

where, “???” is the product id.

And the “******” is the serial number for the touchscreen.

### Registry keys for USB

#### Enable / Disable Touch

Name: 		PacketEnable

Data Type: 		DWORD (REG_DWORD)

Explanation:

Touch is enabled / disabled depending on this value.

Possible Values:

1: Enabled

2: Disabled

### Full Screen Bounding Rectangle and Bounding Mode

Name: 		FullScreenVmode

Data Type: 		DWORD (REG_DWORD)

Explanation:

Defines the mode for the touchscreen boundary in the virtual desktop mode.

Possible Values:

Default is 2.

0: Disable virtual desktop. Touch on all screens will reflect on the primary monitor.

1: Enable virtual desktop. In this mode no individual screen bounds are used for the monitor. Touches toward the edge may generate clicks on adjacent monitors.

2: Enable virtual desktop. Screen bounds are enabled for each monitor. Touches on the screen always generate clicks on the associated monitor. A touch outside the bounding rectangle will cause the cursor to move to a point along the boundary that is nearest to the point of touch.

3: Enable virtual desktop. Screen bounds are enabled for each monitor. Touches on the screen always generate clicks on the associated monitor. Touches outside the bounding rectangle are not sent to the system.

Name: 		FullScreenBounds

Data Type: 		Array of DWORDs (REG_BINARY)

Explanation:

Defines the full screen bounding rectangle in Windows virtual coordinates for the screen.

#### Calibration

The following defines the calibration data used to convert touches from the Elo coordinate system to Windows virtual coordinate system.

The driver uses the following equation to convert a raw touch coordinate point from Elo coordinate system to the Windows screen coordinate system.

Calibration equation is

Xcal = a + m*Xuncal,

where,

Xuncal, is in Elo coordinate system

Xcal, is in Windows coordinate system

m = X_ScrDx / X_EloDx

"a", is the X offset value entry

X_ScrDx = distance between targets in virtual coordinates

X_EloDx = distance between targets in Elo coordinates.

Name: 	X_EloDx, X_ScrDx, X_Offset, Y_EloDx, Y_ScrDx, Y_Offset, Z_EloDx, Z_ScrDx, Z_Offset

Data Type: 		DWORD (REG_DWORD)

Explanation:

Data required by calibration. Use the equation above to calculate these values.

Name: 	SwapXY

Data Type: 	DWORD (REG_DWORD)

Explanation:

Specifies if the touchscreen is rotated 90 degrees or 270 degrees.

Possible Values:

Set to 1 if touchscreen is rotated 90 degrees or 270 degrees else set to 0.

Name: 	WindowsMonitorNumber

Data Type: 	DWORD (REG_DWORD)

Explanation:

Windows monitor number.

Possible Values:

Windows monitor numbers are 1 based.

Name: 	X_Resolution

Data Type: 	DWORD (REG_DWORD)

Explanation:

Screen width in pixels.

Name: 	Y_Resolution

Data Type: 	DWORD (REG_DWORD)

Explanation:

Screen height in pixels.

Name: 	xVirtScrSize

Data Type: 	DWORD (REG_DWORD)

Explanation:

Width in pixels of the Windows virtual screen.

Name: 	yVirtScrSize

Data Type: 	DWORD (REG_DWORD)

Explanation:

Height in pixels of the Windows virtual screen.

Name: 	xVirtScrCoord

Data Type: 	DWORD (REG_DWORD)

Explanation:

Top left X coordinate of the Windows virtual screen.

Name: 	yVirtScrCoord

Data Type: 	DWORD (REG_DWORD)

Explanation:

Top left Y coordinate of the Windows virtual screen.

Name: 	xMonLocn

Data Type: 	DWORD (REG_DWORD)

Explanation:

Monitor location (X) for this monitor on the virtual desktop.

Name: 	yMonLocn

Data Type: 	DWORD (REG_DWORD)

Explanation:

Monitor location (Y) for this monitor on the virtual desktop.

### Edge of touchscreen cursor acceleration

Name: 		EdgeAccelerationEnable

Data Type: 		DWORD (REG_DWORD)

Explanation:

Enables / disables acceleration.

Possible Values:

Default is 0.

0: Disable acceleration.

1: Enable acceleration.

Name: 		EdgeAccelerationBounds

Data Type: 		Array of DWORDS (REG_BINARY)

Explanation:

Rectangle bounds (XMin, YMin, XMax, YMax) beyond which cursor movement is accelerated. Within the defined rectangle touch is interpreted normally. XMin, YMin, XMax, YMax are defined in Windows virtual screen coordinates.

Possible Values:

Name: 		EdgeAccelerationScale

Data Type: 		DWORD (REG_DWORD)

Explanation:

Acceleration value.

Possible Values:

Default is unused.

It can be anywhere in the range of 0-100. 10 indicates no acceleration.

### Special Features

#### Clipping Rectangles

Name: 		VirtualDeskMode

Data Type: 		DWORD (REG_DWORD)

Explanation:

Defines the mode for the clipping rectangles for the screens. Every screen can have a maximum of 64 rectangles overlapping or disjoint.

Possible Values:

Default is not used.

2: Clipping at rectangle bounds is enabled. A touch outside the bounding rectangle will cause the cursor to move to a point along the bounding rectangle that is nearest to the point of touch.

3: Clipping at rectangle bounds is enabled. Touches outside the bounding rectangle are not sent to the system.

Name: 		ExclusionFlag

Data Type: 		DWORD (REG_DWORD)

Explanation:

Defines if touch is enabled within the rectangles or outside the defined rectangles.

Possible Values:

Default is 0.

0: Inside the rectangle.

1: Outside the rectangle.

Name: 		Num_Bounds

Data Type: 		DWORD (REG_DWORD)

Explanation:

Count of rectangles defined in “ClippingBounds” array.

Possible Values:

Default is 0.

0: Minimum Rectangles.

64: Maximum Rectangles.

Name: 		ClippingBounds

Data Type: 		Array of DWORDs (REG_BINARY)

Explanation:

Rectangle boundary values defined for clipping on the screen.

#### Smartset commands issued during driver startup

Name: 		SmartSetCommands

Data Type: 		Array of CHARs (REG_BINARY)

Explanation:

This is used for initializing the controller at start time. If this key is present in the registry then the default initialization in not used. This is an array of smartset packets, where each packet is 8 characters. The driver reads 8 chars in a loop till the end and sends them to the controller at initialization. Please refer to the smartset manual for smartset commands.

Possible Values:

By default this key is not present in the registry. It has to be explicitly created.

#### Reserved

Name: 		ControllerType

Data Type: 		DWORD (REG_DWORD)

Explanation:

This is reserved for internal use.

Possible Values:

Name: 		EnableQuickTouch, QuickTouchDx, QuickTouchDy

Explanation:

Not used.

### Registry keys for HID

#### Touch Mode

Name: 		MouseMode

Data Type: 		DWORD (REG_DWORD)

Explanation:

Touchscreen mode.

Possible Values:

Default is 6.

0: Click on touch.

1: Click on release.

6: Mouse emulation.

Name: 		EnableQuickTouch

Data Type: 		DWORD (REG_DWORD)

Explanation:

Enables / disables quick touch mode.

Possible Values:

Default is 0

0: Disabled

1: Enabled

Name: 		QuickTouchDx

Data Type: 		DWORD (REG_DWORD)

Explanation:

X distance in Windows virtual coordinates outside which quick touch is enabled. Within this distance two quick touches are interpreted as regular touches.

Name: 		QuickTouchDy

Data Type: 		DWORD (REG_DWORD)

Explanation:

Y distance in Windows virtual coordinates outside which quick touch is enabled. Within this distance two quick touches are interpreted as regular touches.

Name: 		UntouchDelay

Data Type: 		DWORD (REG_DWORD)

Explanation:

“UntouchDelay” defines the time in seconds for the timeout. If there is constant touch at the same location a Button Up event will be generated automatically after the timeout period expires.

Possible Values:

Default value is 10 seconds.

Name: 		SwapButton

Data Type: 		DWORD (REG_DWORD)

Explanation:

Swaps the mouse left and right buttons. The buttons remain swapped for the count of touches this value is set to. It remains swapped until this count is decremented to 0. After that, the buttons are automatically swapped back to their original state.

Possible Values:

By default this key is not present in the registry. When the DLL API feature is used the mouse buttons are swapped and this value is added.

Name: 		LeftHandedUser

Data Type: 		DWORD (REG_DWORD)

Explanation:

This is applicable for left-handed mouse users.

Possible Values:

Default is 0

0: Disabled

1: Enabled

Name: 		RightButtonActive

Data Type: 		DWORD (REG_DWORD)

Explanation:

Indicates that the right button application, should be run at user logon for this screen.

Possible Values:

1: Run right button at user logon.

2: Do not run right button for this screen at user logon.

#### Beep On Touch

Name: 		BeepFlag

Data Type: 		DWORD (REG_DWORD)

Explanation:

Beep is enabled / disabled depending on this value.

Possible Values:

0: Disabled

1: Enabled

Name: 		BeepTime

Data Type: 		DWORD (REG_DWORD)

Explanation:

Defines the duration of beep on touch in milliseconds.

Possible Values:

Default is 0x0014

Name: 		BeepFreq

Data Type: 		DWORD (REG_DWORD)

Explanation:

Frequency of beep on touch in Hz.

Possible Values:

Default is 0x00300

#### Drag Delay

Name: 		DefaultDragDelay

Data Type: 		DWORD (REG_DWORD)

Explanation:

Drag delay value in milliseconds.

Possible Values:

Default is 20

Name: 		MinDragDelay

Data Type: 		DWORD (REG_DWORD)

Explanation:

Minimum drag delay value on the scale in Control Panel in milliseconds. Reserved for use by control panel and driver.

Name: 		MaxDragDelay

Data Type: 		DWORD (REG_DWORD)

Explanation:

Maximum drag delay value on the scale in Control Panel in milliseconds. Reserved for use by control panel and driver.

### Special Features

#### Button Sequence

Use the EloBtnSeq application to set these values.

Name: 		Buttons_Touch

Data Type: 		Array of WORDs  (REG_BINARY)

Explanation:

Defines the sequence of mouse button events generated when user touches the screen (corresponding to Button Down).

Name: 		Buttons_Move

Data Type: 		Array of WORDs  (REG_BINARY)

Explanation:

Defines the sequence of mouse button events generated on Drag (Move).

Name: 		Buttons_UnTouch

Data Type: 		Array of WORDs  (REG_BINARY)

Explanation:

Defines the sequence of mouse button events generated when user lifts his finger (Release / Button Up).

# User Specific Registry Keys

User specific configuration data are saved in this key. User key is located in

HKEY_CURRENT_USER\Software\EloTouchscreen

Name: 		DoubleClickHW

Data Type: 		DWORD (REG_DWORD)

Explanation:

Defines the double click width and height.

Possible Values:

Default value is 25.

Name: 		EloTTray

Data Type: 		DWORD (REG_DWORD)

Explanation:

This determines if Tool tray application should be run for this user or not when the user logs in.

Possible Values:

Default value is 1.

1: Run Elo tool tray app

2: Do not run Elo tool tray app.

Name: 		RButton

Data Type: 		Array of DWORDs(REG_BINARY)

Explanation:

Saves x,y Location of Right button application in Windows virtual coordinates.